Health and Safety

Product Data Sheet

SODIUM CHLORIDE EVAPORATED SALT

(GRANULAR SALT & SALT NUTS)

# Stability/Reactivity

Hazardous Reactions: Reaction with concentrated acid will produce hydrogen chloride. Under wet conditions, will corrode many common metals, particularly iron, aluminium, and zinc.

Storage

Keep bags dry. Keep away from concentrated acids.

# Handling

Avoid prolonged skin contact. Avoid inhalation of high concentrations of dusts. Keep away from concentrated acids and common metals. Static electricity can be generated by pneumatic conveying, therefore pipes should be bonded and earthed, especially where a spark could prove hazardous.

# Personal Protection

Wear suitable protective clothing.

# Spillage/Accidental Release

Clear up spillages. Transfer to a container for disposal, alternatively, drench spillage with water and wash to drain.

# Waste Disposal

Disposal should be in accordance with local, state or national legislation.

# First Aid Measures

Inhalation: Unlikely to be required but if necessary treat symptomatically.

Skin Contact: Wash the skin with water.

Eye Contact: Irrigate with eyewash solution or clean water, holding the eyelids apart, for at least 10 minutes. If symptoms develop, obtain medical attention.

Ingestion: Vomiting is likely. Wash out mouth with water and give 200-300 ml (half a pint) of water to drink. Obtain medical attention especially if vomiting has not occurred.

Further Medical Treatment: Unlikely to be required but if necessary treat symptomatically.

# Fire and Explosion

Non-flammable. Will withstand temperatures up to its melting point and beyond without decomposing.

# Health Hazard: Toxicity Data

Inhalation: High concentrations of dust may be irritant to the respiratory tract.

Skin Contact: Dry salt and concentrated solutions will remove the natural greases from the skin resulting in dryness. Repeated and/or prolonged contact may cause irritation.

Eye Contact: High concentrations may cause irritation.

Ingestion: The swallowing of small amounts is unlikely to cause any adverse effects. Excessive doses may result in irritation of the gastrointestinal tract leading to nausea vomiting and diarrhoea.

Long Term Exposure: Chronic effects may result from the ingestion of excessive amounts of either salt or brine. Ingestion of hypertonic solutions can cause disturbance of body electrolyte and fluid balance.

# Regulatory Information

User: Not classified as hazardous to users.

EEC Classification: Under the Classification, Packaging and Labelling of Dangerous Substances regs. 1984, this material is not dangerous for supply or conveyance.

Transport: Not Classified as Hazardous for Transport.