Health and Safety

Product Data Sheet

# NATURAL AGGREGATES

## Product

Crushed rock and sand and gravel aggregates. A separate data sheet is available for finely divided mineral fillers and powders.

## Application

Aggregates are used as basic construction materials and as components of other materials such as concrete, mortar, macadams and asphalts.

## Physical and Chemical Characteristics

Natural aggregates are produced from naturally occurring rock and sand and gravel deposits and consist of combinations of various minerals including quartz.

## Main Hazards

If inhaled in excessive quantities over extended periods, respirable dust containing quartz can constitute a long-term health hazard. Advice on the quartz content of the dust produced from individual aggregates is available via the supplying unit if required.

## Precautions

Inhalation of any dust produced from aggregates should be avoided. Dust masks to BS 2091 type B, or their equivalent, should be worn in enclosed spaces where the handling or further crushing of dry aggregates containing quartz is taking place and where adequate ventilation is not provided.

## Protective Clothing

No special protective clothing is required.

## Transportation, Storage and Disposal

The carrying of aggregates is not subject to hazardous substance conveyance regulations and vehicle labelling is not required. Aggregates are inert but should be disposed of in accordance with local legal requirements.