Health and Safety

Product Data Sheet

Play Sand (Silica Sand)

Product Play Sand (Silica Sand). Washed/Washed and Dried. Main Applications glass, foundry & leisure use.

# Composition 80 – 99.5% (S102)N. Substance type – Naturally occurring (Quartz). Physical status – Solid, granular, odourless. Cas No. 14808-60-7. Einecs No. 238-878-4.

# Hazards Identification As required by the classification. Packaging and Labelling of Dangerous Substances Regulations 1984 (UK):

# Play Sand (Silica Sand) is not labelled or listed as a chemical and presents no respiratory hazard. Silica Flour is labelled and classified as harmful in the minus 5 micron range i.e. dust. Homepack do not supply this product. Respirable dust, the smaller dust fractions found in silica flour may if inhaled over long periods lead to lung damage, particularly silicosis. Larger sand particles are unlikely to be inhaled into the respiratory system. Dust exposure may occur where transfer from one process to another is involved.

# First Aid Measures No immediate short term effects. Non irritant but eye contact should be avoided. Eye protection recommended and I contact occurs wash out with water. Ingestion should be avoided.

Fire – Fire Measures Non flammable – no fire or explosive hazard.

Accidental Release Measures In the event of spillage or leakage occurring, dry sweeping should be avoided. Water spraying or ventilated vacuum systems should be used. Exposure limit for sand drying is 150 MG/M3

Physical and Chemical Properties Density – 2.635…….2.660 g/cu.cm. Water Solubility – Insoluble. Meeting point – 1610oC Boiling Point – 2230oC

# Stability and Reactivity Non reactive, stable in water and soil mediums. Non photo-degradable. Non oxidising, soluble in hydrofluoric acid.

# Toxicological Information  Toxicity data arising from tests involving humans is inconclusive and there is no evidence available, at present, to justify control measures based on carcinogenicity. Play Sand (Silica Sand) is not classifiable as to its carcinogenicity to humans.

Ecological Information No data exists on toxicity to fish, aquatic invertebrates, aquatic plants, micro organisms, terrestrial plants or soil swelling organisms.

# Disposal Considerations By means of waste authority approved waste disposal method for solid inert waste.

Transport Information Non hazardous. Vehicle labelling is not required.

Regulatory Information This data sheet is structured along the recommendations given in schedule 6 of the Chemical Hazard Information and Packaging Regulations 1994. The Health and Safety at Work Act 1974 and the Control of Substances Hazardous to Health Regulations 1991. Play Sand (Silica Sand) is not classified as a chemical hazard.

Other HSE’s guidance note EM59 “Crystalline Silica” draws attention to the Health risks that may result from exposing to respirable silica and lists further references.

The information given in this document has been compiled on the basis of the best available knowledge. It does not imply that the information is complete, neither does it constitute any warranty, expressed or implied, as the quality, properties, conditions or otherwise of the product. It is the users responsibility to satisfy themselves as to the application of the information and the correct use of the product supplied.